# Formulary EE

| Charge and amounts of charge |  |
| --- | --- |
| Proton Charge symbol (“elementary charge”) | e |
| Electron charge symbol | -e |
| Symbol for amounts of charge (aka “charge”) Unit of amounts of charge Relations | Q: constant amount of charge q(t): time variable charge [Q] = [q] = Coulomb = C 1 C=6.24∙1018e 1 e=1.60∙10-19C -e=-1.60∙10-19C 1C=1As |
| Electric current |  |
| Amount of charge passing through a cross-section in the indicated direction during the time | Q A t |
| Electric current magnitude (aka current), mean value Instantaneous electric current Unit of the electric current Relations | I=Q/t it=dq(t)/dt I=i=Ampere=Amp=A 1 A=1Cs 1C=1As 3600 C=1 Ah |
| Current direction |  |
| “real current (electron) direction” “conventional direction of current” |  |
| Current density |  |
| Cross sectional area | A |
| Current vertical through cross-sectional area | I |
| Current density, mean value: | J=IA |
| time-dependant current density Unit of current density Example: Copper cables | jt=i(t)/A J=A/m2 about+5°C at 6A/mm2 (no heating) about +70°C at 16 A/mm2(for higher Temps) |

| Power and energy |  |  |
| --- | --- | --- |
| Formula symbol for average power Instantaneous power Unit of Power |  | P p(t) P=p=Watt=W |
| Symbol for energy, from “work” - E is reserved for electric field strength Unit of energy |  | W W=Joule=J 1J=1Ws=1Nm=1kg∙m2s2 1kWh=3.6∙106J |
| Voltage |  |  |  |
| Definition of voltage (aka electromotive force) “ratio of the energy released to the amount of charge transported from point A to point B” Symbol, mean value Time variable voltage Unit voltage Relations | UAB=∆W/∆Q →W=U∙Q=V∙[As or C] U, (USA: V) u(t) U=JC=Volt=V 1 V=1J/As |  |  |
| Nominal voltage = average voltage = (Fully charged – allowable discharge)/2 | The nominal voltage allows the energy stored in a fully charged battery to be calculated, assuming a linear voltage decrease during discharge. |  |  |
| Resistance and Conductance |  |  |  |
| Definition of electrical resistance Unit of resistance | R=U/I U=R∙I I=U/R R=VA=Ω=Ohm |  |  |
| Definition of electrical conductance Unit of conductance: | G=I/U G=AV=S=Siemens=aka mho (US.) |  |  |
| Resistor: const. ratio between voltage and current | UI=const |  |  |
| Specific Resistance, Specific Conductance |  |  |  |
| Specific resistance “rho” use: calculate resistance of a wire based on its geometrical properties and ρ Unit | ρ=A∙Rl R=ρ∙lA ρ=Ω∙m=10-6∙Ωmm2m |  |  |
| Specific conductance “sigma” Unit | σ=1ρ σ=Sm |  |  |
| Power |  |  |  |
| Average power | P=∆W∆t=∆Q∆t∙U P=I∙U=R∙I2=Watt |  |  |
| Instantaneous power | pt=ut∙i(t) |  |  |
| Kirchhoff’s laws |  |  |  |
| #### Mesh rule The sum of all voltages in a mesh, taking into account the direction of the arrows, is 0. Justification: energy conservation | #### Node rule The sum of all currents towards a node is equal to the sum of all currents flowing away from the node. Reason: the ideal node is a pure connecting element and therefore cannot store any charge. |  |  |
| Systematic error correction |  |  |  |
| URX=Um-UA=Um-IRX∙RA=Um-Im∙RA | IRX=Im-IV=Im-UmRV |  |  |
| Ohmic Resistors |  |  |  |
| Series |  |  |  |
| Parallel |  |  |  |
| Ohmic voltage divider | Ohmic current divider |  |  |
| U1U2=R1∙IR2∙I=R1R2 U2=R2∙I=R2∙UR1+R2=R2R1+R2∙U | I2=UR2=I∙(R1∙R2R1+R2)R2=R1R1+R2∙I I1I2=UR1∙R2U=R2R1 |  |  |

Linear model for batteris and accumulators

«Linear voltage source» = ideal voltage source + series resistance (“Internal resistance Ri”) Ri=U0I0

Network Analysis

Conditions for the method (Superposition Principle)

- Linear network - Allowed as used elements are resistors, linear sources, (capacitors, coils) - There is only one source in each partial solution - The current and voltage arrows on a particular one-port must point in the same direction in all partial solutions, preferably defined at the beginning.

Superposition Principle

- all sources except 1 = 0 calculate currents & voltages - next source to value, all others = 0 calculate currents & voltages - add up partial solutions - Ideal voltage source with 0V = short circuit! - Set

| Batteries in Parallel |  |
| --- | --- |
| Paralleling batteries results in higher available currents | U0= U1∙Ri1+U2∙Ri2Ri1+Ri2 Ri= Ri1∙Ri2Ri1+Ri2 I0=U1Ri1+U2Ri2 |
| Batteries in Series |  |
|  | U0=U1+U2 Ri=Ri1+Ri2 I0= U1+U2Ri1+Ri2 Paralleling similar batteries results double the voltage, whereas the short-circuit current stays the same |
| Exercise 4 – Task 1 | Solution |
| One of the first cars made by Tesla, the Tesla Roadster, used a battery bank consisting of 6831 batteries type 18650. Let us assume, all those batteries have an idle voltage of 3.7 V and an internal resistance of 100 mΩ. The batteries are connected as follows: Groups of 69 batteries are connected in parallel All these 99 groups are connected in series | Due to all batteries having the same idle voltage of 3.7 V and the same internal resistance of 100 mΩ, calculations are straightforward: - A group of 69 batteries in parallel has an internal resistance of 100 mΩ/69 = 1.45 mΩ and an short circuit current of 69 ꞏ 3.7 V / 100 mΩ = 2553 A - All groups in series, i.e., the equivalent battery, has an internal resistance of 99 ꞏ1.45 mΩ = 143.6 mΩ and an idle voltage of 99 ꞏ 2553 A ꞏ 1.45 mΩ = 366.5 V. |
| Temperature model for metal alloy resistors |  |
| α20 Linear Temperature coefficient (in 1/Kelvin or 1/°C) β20 Quadratic Temperature coeff. (in 1/Kelvin2 or 1/°C2) R20 Resistance value at 20 °C Δϑ Temperature increase Rϑ Value of resistance at the temperature ϑ of the element (possibly ≠ ambient temperature), [ϑ] =°C | Polynomial description : Rϑ=R20∙(1 + α20∙∆ϑ + β20∙∆ϑ2) If β20 is not given, you can neglect it |
| NTC temperature model |  |
| T Temperature in Kelvin R Value of resistance at this temperature R25 Nominal resistance at 25 °C A1,B1,C1,D1 Coefficients (see datasheet) | T(R) = 1A1+B1lnRR25+C1(lnRR25)2+ D1(lnRR25)3 |

| Magnetic Force Between Conductors |  |
| --- | --- |
| μ depends on the material between the conductors, it is called “permeability” | Fm=I1∙I2∙+ l∙μ∙12πr=[N] |
| Permeability μ |  |
| Unit of permeability Permeability of the vacuum Permeability of any substance Relative permeability | μ= V∙s/(Am) μ0= 4 ∙π∙10-7VsAm μ μr= μ/μ0 |
| Magnetic Field Strength |  |
| “Magnetic field strength B1” of a long straight current I1 at a distance r | B1=μ0∙I12πr |
| Magnetic field strength B | Magnetic field strength H |
| also “magnetic flux density”, “magnetic induction” Unit: B=V∙sm2=Tesla=[T] | also “magnetic excitation” Unit: [H] = A/m |
| Relationship: H = Bμ |  |
| Application of Magnetic Field Strengths |  |
| Vector length: B = μ0∙I2πr | FM=I2∙l⋅B1 FM=I∙l∙ B∙ sin α For an angle α≠ 90° |
| Magnetic Field Vector |  |
| Definition For force as a vector, a current vector and a magnetic field (strength) vector is needed, and the ordinary product is converted to a cross product: | FM= l∙ I2 ×B1 |
| Magnetic Field Strength Inside a Coil |  |
| for current I, Length ℓ, diameter d, Number of turns N, inside a coil | B = μ∙I∙Nl2+d2 FM= N∙I ∙l∙B |

| Magnetic flux |  |
| --- | --- |
| Unit of magnetic flux B homogeneous with inclination angle α referred to a flat surface A | [ϕ] = T∙m2= Vs = Weber = Wb ϕ=B∙A∙sinα ϕt=B∙A∙cos2∙π∙t1f |
| Induced voltage |  |
|  | u = -dϕdt if ut=dϕdt=const.=umax=dBdt=d(B∙l∙v∙t)dt=B∙l∙v |
| Linked flux |  |
| Linked flux “Psi” for coil with N turn with the flux ϕ (magnetic flux) per turn [Wb] | ψ= N∙ ϕ ϕ=B∙A |
| Linked voltage |  |
| Max voltage P: number of poles, f: frequency | u = - dϕdt∙N = -dψdt maxu=4 ∙p∙ψpole∙π∙f |
| Coil with iron core |  |
| Higher field strength with μr > 300 | B = μ0μr∙I∙Nl2+d2 |
| Eddy current | Magnetic field of the eddy current |
| Eddy current, ideal coil without resistance connected to an ohmic resistor R in an external magnetic field B(t), causing linked flux ψ(t). i = - dψdt/R | Eddy current causes a field in the coil Field direction according to right-hand rule Bi = μ∙I∙Nl2+d2 |
| Self induction |  |
| Unit: L= Henry =H=[Wb/A] Self-induced voltage with inductance L | uL =L∙didtd: Differenz,ϕ= LN∙i |
| Cylindrical coil |  |
| - Length l - Diameter d - Windings N - Cross-sectional area A Energy in the coil | L = μ0μr∙Al2+d2∙N2=[H] WL= 12LI2=[J] |
| Toroidal coil for thin wire |  |
| - d: Core diameter - r: Inner radius - R: Outer radius - N: windings | L = N2 ∙μ0μr∙d2π∙lnRr=[H] |
| Wire resistance | uS=L∙didt+Rs∙i, uL=L∙didt |
| PM Motor torque |  |
| For a single loop, with current I, side length ℓ, lever arm r, magnetic field B, angle α between surface normal and magnetic field With current I, A surface area, B magnetic field and N windings Torque with many loops, without mechanical losses, with torque constant kM | Unit of torque [M] = Nm Unit of the torque constant [kM] = Nm/A M = 2 ∙I ∙l∙B∙r∙sin α M = I∙A∙B∙N M = kM∙I=[Nm] kM = MI=NmA |
| PM Motor: Equivalent Circuit, Symbol |  |
| For the PMDC motor UBEMF= n /kn with speed constant kn and speed n, [n] = min-1= rpm Unit of the speed constant but sometimes also | UBEMF= n/kn kn=min-1V =rpmV kn=1000∙min-1V |
| Motor Power |  |
| Motor Power, voltage U at motor, current I through the motor | Pin= U∙I |
| Mechanical Power (without mechanical losses) ω = 2 π n60 | Pout= M∙ω= M ∙n∙π30 |
| Efficiency “eta” | η= PoutPin |
| Relationship between the torque kM· kn= 30 / π constant and the speed constant when mechanical losses are neglectable | kM∙kn= 30π kn = 30π∙kM [rpm/v] |
| Motor Characteristics of a PMDC Motor |  |
| M = kmRs∙U0-n∙kmkn∙Rs∙ | n = kn∙U0-knkm∙Rs∙M |
| Idle speed [rpm] | n0=kn∙U0 |
| Holding torque / stands still | MH=kmRs∙U0 |

| Capacitance |  |
| --- | --- |
| Capacitance: Two capacitors connected in series have always the same charge! N=nano, u=my=micro Unit Relations to the unit: | C = QCUc [C] = Farad = F 1 F = 1 C/V = 1 A/V |
| Charge Charge when parallel | Qc= C ∙Uc Q = Q1+Q2 |
| Instantaneous charge | qct=C∙uct |
| Instantaneous current | ic(t)=C∙ducdt |
| Momentary voltage | uc(t)= 1Ct0tic∙dt +uc(t0) |
| Energy in a capacitor | W = 12∙Qc∙Uc W = 12∙C∙UC2 W = 12QC2C |
| General formula for capacitors in series 2 series capacitors (only for two cap.) | 1Ctot=1C1+1C2 Ctot= C1∙C2C1+C2 |
| 2 Parallel capacitors | Ctot=C1+C2 |

| Dielectric strength |  |  |
| --- | --- | --- |
| Dielectric strength Unit Flashover voltage d: dielectric thickness Maximum working voltage | ED ED=kVmm UD=ED∙d UCmax≪UD |  |
| Coulomb’s Law, Permittivity |  |  |
| Attraction/repulsion force for two charged spheres at distance d (d >> sphere radius R) Permittivity (≠permeability) of a material Unit of permittivity | F = 14πεQ1∙Q2d2 ε ε=Fm |  |
| Permittivity of the vacuum | ε0= 8.85 ∙10-12 F/m |  |
| Dielectric constant (“epsilon r”) | εr= ϵ/ε0 or ϵ=εr∙ε0 |  |
| Flashover voltage | Umax= dialectric strength∙thickness |  |
| Capacitance Formulas |  |  |
| Plate capacitor Distance d or thickness, area A If two plates -> 2 windings -> multiplicate by 2 |  | C = εAd=εrε0Ad |
| Spherical capacitor Outer radius ra, Inner: ri Free standing sphere with radius r |  | C = 4πε1r1-1ra C = 4πεr |

| AC |  | Bei Mix von DC und AC -> DC berechnen -> î und Û berechnen und dann weiter |
| --- | --- | --- |
| Definitions about AC |  |  |
| Amplitude Û (maximum value), angular frequency ω, time t, zero phase angle: φu, by convention -180°<φu ≤180° |  | u(t) = Ûcos(ωt+φu) |
| Frequency f |  | ω = 2πf = 2πT [ω] =rad/s |
| Average power |  | P = Û∙Î2 |
| Instantaneous power |  | p(t) = u(t)∙i(t) |
| RMS value of a voltage Amplitude |  |  |
|  |  | P = Û∙Î2=Û∙Û2∙R= Û22R=U2R→U=Û2 |
| RMS voltage Û= Amplitude |  | URMS=Û2 |
| RMS current (meaning root-man-squared) Î = Amplitude |  | IRMS=Î2 |
| Capacitors at alternating current |  |  |
|  | i = C dudt u = 1Ci dt |  |
| Ratio current/voltage at the capacitor |  |  |
| Zc="Impedance of the capacitor” (always positive, unit: Ω) u(t)=Ûsin(ωt) i(t) = Îcos(ωt)=C∙du(t)dt=C∙Ûcos(ωt)∙ω | →Î = C∙Û∙ω →ÛÎ=UI=1ω∙C |  |
| Coils at alternating current |  |  |
|  | u = LddTi i = 1Lu dt |  |
| Ratio current/voltage at the coil |  |  |
| Coil impedance always positive, unit:Ω it=-îcosωt u(t) = Ûsin(ωt) = L∙di(t)dt=L∙Îsin(ωt)∙ω | →Û=L∙Î∙ω →ÛÎ=UI= ω∙L = ZL |  |
| More AC quantities |  |  |
| Admittance | Y = 1Z [Y ] = Siemens = S |  |
| Capacitor reactance* Minus sign to distinguish capacitive/inductive (convention) | Xc=-Zc=-1ω∙C |  |
| Capacitor susceptance | BXc=1Zc=-1Xc=ω∙C |  |
| Coil reactance | XL=ZL=ω∙L |  |
| Coil susceptance | BL= -1ZL=-1XL=-1ω∙L |  |
| Conductance | G = 1R |  |

| AC Circuits and calculations series |  |
| --- | --- |
| AC Circuits and calculations parallel |  |

Conversion Scale						Areas

Circle: A=πr2=d2∙π4

Integration

abf(x)dx =F(b)-F(a)=[F(x)]ab= F(x)|ab

Definition

Eine Funktion F(x) heisst Stammfunktion von f(x), falls F'(x) = f(x).

Bsp: f(x) =x2⇒ F(x) = 13x3

Bsp: 13x2-4x dx=13x3-2x213=1313-2∙32-1313-2∙12=-223

Definition unbestimmtes Integral

- Die Menge aller Stammfunktionen = unbestimmtes Integral von f(x)

- f(x)dx.

- Die Funktion f(x) = Integrand.

Beispiel:

4x2- 2x+3dx =43x3 - x2+3x+C